C64 Mechanical Keyboard & Kernal Switcher

Standard Version Rev. 0

Functional Description

SW1 to SW66, with the exception of SW35, form the keyboard matrix of the C64 keyboard. It is identical to the original C64 keyboard matrix.

SW17A, SW18A, SW32A, SW33A, SW49A, SW51A, SW62A and SW65A are alternative positions for 1.5U keys, which nowadays typically have a central stem. They are wired in parallel with their corresponding classic-position switches (without the “A”).

SW32 is the RESTORE button. It is connected to the /RESTORE pin and GND, just like in the original keyboard. Unlike the original, it has a 10 kΩ pull-up resistor (R15), since the RESTORE signal is also required to have a defined state when the keyboard is not connected to a C64 mainboard.

SW35 is the SHIFT LOCK button. Unlike the original keyboard, where it is a latching switch and is electrically connected in parallel with the left SHIFT key, SW35 is connected to a dedicated Shift Lock circuit.

R3 is the pull-up resistor, and C1 provides switch debouncing. R4, Q2 and R5 form an inverter, so that the LOCK signal goes high while the SHIFT LOCK button is pressed.

C2 and R6 form the power-on reset circuit for the SHIFT LOCK flip-flop IC1A. This works perfectly on a breadboard, but fails when the circuit is connected to an actual C64. This is why a modification wire (“botch wire”) is required between IC1A pin 1 and the /C64RES reset signal.

IC1A toggles its output state every time the SHIFT LOCK button is pressed. R2 and Q1 drive the SHIFT LOCK LED (SW35LD), while R1 is the current-limiting resistor.

The value is calculated as follows:

R1=5V-VF,SW35LDIF,SW35LD

VF is the forward voltage of the LED (approximately 2 V for a red LED; see the datasheet). IF is the forward current, which should be below the maximum current specified in the LED datasheet. 100 Ω is the minimum resistor value for an LED with a maximum forward current of 30 mA.

The diode D1 blocks any reverse current from the base of Q3. Q3 is the SHIFT LOCK switch. This circuit is derived from the SX-64 I/O board schematics, which also does not use a latching SHIFT LOCK switch. It pulls ROW7 low while COL1 is pulled low by the C64 keyboard scan. This is electrically in parallel with the left SHIFT button, SW51.

J1 is a pin header that connects to the ribbon cable between the C64 keyboard and the C64 Keyboard Connector PCB.

M2 is the Pro Micro microcontroller module. It provides the advanced functions of the keyboard.

SW67 (PREV) and SW68 (NEXT) are the two additional keys that occupy the space to the left and right of the 7U space bar. They cannot be populated when a traditional 9U space bar is used.

Q5 is the amplifier for the piezo buzzer. D2 is the freewheeling diode for the piezo buzzer, which can produce a voltage spike when subjected to mechanical impact.

J4 is the jumper that must be installed for a short-board C64. R10 is the pull-up resistor for the /SHORT_BRD signal.

J6 is a reserve connector with unused pins and supply voltage. TX and RX are the signals of the serial port, while A3 is an analog input.

J2 is the connector for the Kernal switcher, providing access to the upper address bits A13–A15 of the EPROM in the Kernal adapter. /C64RES is connected to the internal /RESET signal of a classic C64 mainboard. /EXROM is connected to the /EXROM signal of the classic C64 expansion port.

This is, how an EXROM Reset works:

With /EXROM = high (no cartridge connected), the CPU accesses the RAM instead. The 6510 CPU cannot distinguish between data read from RAM and data read from a cartridge ROM during the cartridge detection process. It therefore searches the first bytes of the cartridge memory area for the CBM80 signature.

Some programs deliberately leave a fake CBM80 signature in RAM. If the CPU finds this signature, it assumes that a cartridge is present and interprets the bytes preceding the signature as the cartridge's start address and NMI vector. These vectors can be manipulated by the program so that, after a RESET, the C64 does not return to the normal reset routine and the program memory cannot be read out normally.

By briefly asserting /EXROM during RESET, the CPU is forced to treat the memory area as cartridge ROM. The resulting cartridge detection process prevents the fake CBM80 signature left in RAM from being effective. This can circumvent a certain type of copy protection that manipulates the RESET and NMI (RESTORE) vectors.

J7 is the external I²C bus. It is intended for connecting a display. R12 and R13 are the pull-up resistors required by the Pro Micro's I²C bus.

J5 connects to the WS2812B RGB LED strip. R16 and C4 are components recommended in some LED strip application notes.

M1 is the GPIO module. It contains a TI PCF8575 I²C GPIO expander, which allows the Pro Micro to scan the keyboard matrix when USB keyboard functionality is required.

If the module is not used, other I²C devices or sensors could be connected to the bus. This would require additional software, however. One possible application would be connecting temperature sensors to monitor the temperatures of various ICs.

A power LED can be connected to either J3 or J8, which are located at opposite ends of the keyboard PCB. R11 is the current-limiting resistor and provides approximately 10 mA for a standard red or green LED.

PWRLED controls the connected LED. It is normally on and is switched off temporarily for signaling purposes. If M2 is not populated, closing JP3 provides a permanent supply for the LED, keeping it continuously on.