C64 Keyboard Connector PCB Rev. 0

Module Description

# Introduction

The purpose of this PCB is to connect the C64 Keyboard (Project. No. 223 or 225) to the C64/Ultimate64 Keyboard Connector.

Figure 1: C64 Keyboard Connector PCB with ribbon cable

The connector for the ribbon cable is a MicroMatch series connector (red). They are low profile, so the cable does not collide with the side of the case when connected to a C64 long board. The C64 Keyboard has a 2x10p pin header, so the ribbon cable has two different types of connectors attached.

# Dimensions

Figure 2: Dimensions of the PCB

# Pinouts

## J1 – Connector to mainboard

2.54mm pitch socket, 90°

| Pin | Signal |
| --- | --- |
| 1 | GND |
| 2 | No pin |
| 3 | /RESTORE |
| 4 | +5V |
| 5 | ROW3 |
| 6 | ROW6 |
| 7 | ROW5 |
| 8 | ROW4 |
| 9 | ROW7 |
| 10 | ROW2 |
| 11 | ROW1 |
| 12 | ROW0 |
| 13 | COL0 |
| 14 | COL6 |
| 15 | COL5 |
| 16 | COL4 |
| 17 | COL3 |
| 18 | COL2 |
| 19 | COL1 |
| 20 | COL7 |

## J2 – Ribbon Cable Connector

MicroMatch Socket, 20 pins

| Pin | Signal |
| --- | --- |
| 1 | GND |
| 2 | No pin |
| 3 | /RESTORE |
| 4 | +5V |
| 5 | ROW3 |
| 6 | ROW6 |
| 7 | ROW5 |
| 8 | ROW4 |
| 9 | ROW7 |
| 10 | ROW2 |
| 11 | ROW1 |
| 12 | ROW0 |
| 13 | COL0 |
| 14 | COL6 |
| 15 | COL5 |
| 16 | COL4 |
| 17 | COL3 |
| 18 | COL2 |
| 19 | COL1 |
| 20 | COL7 |

# Cable

The cable is a 20p ribbon cable of 45cm with one side attached to a 2x10p IDC socket with strain relief on the keyboard side and a 20 pin MicroMatch connector (male) on the side of this PCB. The cable is shown in the /Cables/pdf/ sub-folder of this repository, drawing number 233-3-01-00 (KB_Cab.pdf).

Both connector types are easy to attach, since they are both IDC (insulation-displacement contact), so you only require a vice to compress them, after the ribbon cable is positioned.

The length of 45cm is the longest possible, calculated for a C64 longboard (the keyboard connector is top left on the mainboard). For short boards, 30cm is long enough

# Revision History

## Rev. 0

- Working prototype